SECTION 09 90 00 PAINTS and COATINGS

MASTER

ARCHITECTURAL PAINTING SPECIFICATION

(Parallel Version KM-DE)

The information contained in this Behr Parallel Specification is primarily intended for use in reviewing commercial painting specification submittals. Because coating manufactures may change product formulations, users of this comparative data assume all responsibilities for acceptability of references such as resin types, composition, or gloss levels contained herein. Due to the many variables of Architects’ project painting specifications, it is recommended that the paint manufacturer(s) be contacted for their product’s intended use, technical data, SDS and VOC content compliance with authorities having jurisdiction.

These BEHR PAINTS are in conformance with LEED v4 BUILDING DESIGN AND CONSTRUCTION for LEED v4 BD+C New Construction, Core and Shell, Schools, Retail, Hospitality and Healthcare. All Behr Paints comply with South Coast AQMD Rule 1113. Interior and other Behr Paints are GREENGUARD® Gold Certified.

1801 E. St. Andrew Place │ Santa Ana, California 92705 (714) 545-7101 │ Fax (714) 241-1002

SECTION 09 90 00 PAINTS AND COATINGS

**  NOTE TO SPECIFIER	This Section is for the specification of coatings in a mild to moderate environment.  It does contain some products from Diamond Vogel’s High Performance product lines to allow for design flexibility.  Diamond Vogel recommends the use of Section 09 96 00 High Performance Coatings for areas that require maximum protection.

##### PART 1 - GENERAL

##### 1.01 SUMMARY

Work includes furnishing of materials and equipment, preparation of surfaces and completion of painting and finishing of surfaces as required by Drawings and specified herein.

##### 1.02 RELATED WORK

- Factory, pre-finished items as specified in various sections.

- Shop painting specified in respective sections.

- Architectural woodworking.

- Surfaces not to be painted:

  - Pre-finished wall, ceiling and floor coverings.

  - Items with factory-applied final finish.

  - Concealed ducts, pipes and conduit.

  - Surfaces specifically scheduled or noted on Drawings not to be painted.

##### 1.03 SUBMITTALS

- Product data:

  - Not less than thirty (30) days before beginning work, submit a complete list of materials proposed for use, together with manufacturer's specifications.

  - Paint materials and products shall be subject to Architect's approval.

- Color samples:

  - Prepare color and finishes on samples, 8-1/2" x 11" in size.

  - Submit samples as requested until required sheen, color and texture is achieved.

  - Prepare wood samples on type and quality of wood specified for use on project.

  - Label and identify each sample as to location and application.

##### 1.04 COLORS

- Colors are to be selected or approved by Architect and actual color chips shall be supplied to Contractor for matching. All undercoats shall be tinted to approximately half the color of finish coat.

- Approval of final colors: Do not apply final coat of paint until colors have been approved by Architect.

- The number of colors to be used shall be as determined by Architect. Architect reserves right to vary colors throughout project.

##### 1.05 DELIVERY, STORAGE AND HANDLING

- Deliver paint materials in sealed original labeled containers bearing manufacturer's name, type of paint, stock number, color and instructions for reducing or mixing where applicable.

- Paint materials and equipment

  - Store only acceptable project materials on site.

  - Store in a suitable location.

  - Restrict storage to paint materials and related materials.

  - Comply with health and fire regulations.

##### 1.06 PROJECT CONDITIONS

- Comply with manufacturer's recommendations as to environmental conditions under which coatings and coating systems can be applied. Do not apply paint or coatings when temperature is below 50F. Do not apply exterior paint in damp or rainy weather; ensure that the surface has dried thoroughly before proceeding.  Surface temperature must be at least 5°F above dew point before painting.

- Do not apply finish in areas where dust or contaminants are being generated.

#### PART 2 - PRODUCTS

##### 2.01 MATERIALS

- Basis-of-Design Product: Provide products listed from Behr Paint Company, Santa Ana, California 92705, or comparable products by one of the following. Other manufacturers to conform to materials listed and be approved by Architect.

- Kelly-Moore Paint Company

- Dunn-Edwards Corporation

- Materials selected for coating systems for each type surface shall be the product of a single manufacturer.

- Accessory materials such as thinner, linseed oil, putty and shellac shall be of the highest quality and by approved manufacturer, which comply with local VOC content limits regulations.

- Paints shall be ready-mixed except field catalyzed coatings.

- Do not thin finish coats without Architect's approval.

- Unsuitability of specified products: Claims concerning unsuitability of any material specified or inability to satisfactorily produce the work will not be entertained, unless such claim is made in writing to Architect before work is started.

- Number of coats scheduled is minimum. Apply additional coats at no additional cost if necessary to completely hide base materials, produce uniform color and provide satisfactory finish result.

- Submitted paints and coatings comply with the established VOC content limits of South Coast AQMD Rule 1113.

#### PART 3 - EXECUTION

##### 3.01 EXAMINATION

- Examine surfaces scheduled to receive paint and finishes for conditions that will adversely affect execution, permanence or quality of work and which cannot be put into acceptable condition through preparatory work as included in Article 3.02 "Preparation ". Notify Owner’s Representative in writing of any defects or conditions which will prevent a satisfactory installation.

- Examine surfaces scheduled to be finished prior to commencement of work. Report any condition that may potentially effect proper application.

- Maximum Moisture Content of Substrates: When measured with an electronic moisture meter as follows;

  - Concrete: 12 percent

  - Portland Cement Plaster and Stucco: 12 percent

  - Masonry (Clay and CMU): 12 percent

  - Wood: 15 percent

  - Gypsum Board: 12 percent

- Portland Cement Plaster Substrates: Verify that plaster is fully cured.

- Verify suitability of substrates, including surface conditions and compatibility with existing finishes and primers.

- Proceed with surface preparation and coating application only after unsatisfactory conditions have been corrected.

- Application of coating is construed as acceptance of surfaces and conditions.

##### 3.02 PREPARATION

- Clean surfaces to receive paint thoroughly of substances, which could impair adhesion of paints, including dust, dirt, oil and grease before application of any coatings. Prepare surfaces as follows:

  - WOOD SUBSTRATES: Remove mill glaze and dust, sand smooth. Fill open joints, cracks, nail holes and other pits or depressions flush and smooth with wood filler after priming. Use wood putty to match finish paint coat. Touch up knots or sap streaks with a stain-blocking sealer before priming.

  - CONCRETE, PRE-CAST CONCRETE, TILT-UP: Remove release agents, curing compounds, loose particles, efflorescence and chalk.  Do not coat surfaces if moisture content or alkalinity of surface to be coated exceeds that permitted in the manufacturer’s written instructions. Prime with an alkali-resistant primer.

  - PLASTER and STUCCO SUBSTRATES: Fill hairline cracks, small holes and imperfections on surfaces with patching compound. Smooth off to match adjacent surfaces. Apply an alkali-resistant primer or wash with fresh water and neutralize high alkalinity surfaces where they occur.

  - MASONRY SUBSTRATES: Remove efflorescence and chalk. Do not coat surfaces if moisture content or alkalinity of surface to be coated exceeds that permitted in the manufacturer’s written instructions. Prime with an alkali-resistant primer.

  - SHOP-PRIMED STEEL SUBSTRATES: Solvent clean according to SSPC-SP1. Remove loose primer and rust, contaminants and foreign matter. Prime bare steel and touch up abrasions with a ferrous metal primer.

  - STEEL, FERROUS METAL SUBSTRATES: Remove rust, mill scale, foreign substances and shop primer. Clean according to SSPC-SP3, “Power Tool Cleaning”, or abrasive blasting cleaning according to SSPC-SP6 “Commercial Blast Cleaning” as required. Protect surface from corrosion until application of primer.

  - GALVANIZED METAL: Remove oils, passivators and clean entire surface with an appropriate solvent. Pre-treat with a phosphoric acid etching solution to promote adhesion of subsequently applied coatings.

  - PHOSPHATIZED METAL or GALVANNEALED METAL shall not be chemically etched. Clean and apply suitable metal primer.

  - ALUMINUM SUBSTRATES: Solvent clean according to SSPC-SP1. Remove loose surface oxidation.

  - GYPSUM BOARD SUBSTRATES: Remove dust and foreign matter. Fill pits flush and smooth with joint compound and where required, apply skim coat to provide the required finishing level based on GA-214-96, Recommended Levels of Gypsum Board Finish before application of decoration.

- Surfaces, which cannot be prepared or painted as specified, shall be immediately brought to attention of Architect in writing.

  - Starting of work without such notification will be considered acceptance by the Contractor of surfaces involved.

  - Replace unsatisfactory work caused by improper or defective surfaces as directed by Architect at no additional cost to Owner.

##### 3.03 APPLICATION

- Do not apply initial coating until moisture content of surface is within limitations recommended by paint manufacturer.

- Application:

  - Apply paint with suitable brushes, rollers or spraying equipment.

  - Apply stain in accordance with manufacturer's recommendations.

  - Rate of application shall not exceed that as recommended by paint manufacturer for surface involved.

- Comply with recommendations of product manufacturer for drying time between succeeding coats.

- Leave parts of molding and ornaments clean and true to details with no undue amount of paint in corners and depressions.

- Make edges of paint adjoining other material or color clean and sharp with no overlapping.

- Refinish whole wall where portion of finish is not acceptable.

- Apply materials evenly with appropriate film thickness and free of runs, sags, skips and other defects. Hard, glossy finishes shall be sanded lightly between coats, dusted and cleaned before recoating.

- Remove hardware, hardware accessories, plates, lighting fixtures and similar items in place prior to painting and replaced upon completion of each space.

- Disconnect heating and other equipment adjacent to walls using workmen skilled in appropriate trades, and moved to permit wall surfaces to be painted. Following completion of painting, they shall be expertly replaced and reconnected.

- Paint visible surfaces behind vents, registers or grilles flat black.

  - Wash exposed metal with solvent then prime and paint as scheduled.

  - Spray paint wherever practical.

- Do not paint over Underwriters' labels, fusible links or sprinkler heads.

- EXPOSED PLUMBING AND MECHANICAL ITEMS: Items without factory finish such as conduits, pipes, access panels and items of similar nature shall be finished to match adjacent wall and ceiling surfaces unless otherwise directed.

##### 3.04 CLEANUP

Upon completion of work, remove equipment, excess material and debris, remove paint splatters and leave area in a neat and orderly condition.

##### 3.05 PROTECTION

- Protect work of other trades, whether to be painted or not, against damage by painting. Correct damage by cleaning, repairing or replacing, and repainting, as acceptable to Architect.

- Provide "wet paint" signs to protect newly painted finishes. Remove temporary protective wrappings provided by others for protection of their work after completion of painting operations.

  - At completion of construction activities of other trades, touch up and restore damaged or   defaced painted surfaces.

##### 3.06 FINISH SCHEDULE

- Finish surfaces in accordance with schedule. Catalog names and numbers refer to products as manufactured by the Behr Paint Company, Santa Ana, California 92705, except as otherwise specified.

- Provide paint finishes of even uniform color, free from cloudy or mottled appearance. Properly correct non- complying work to satisfaction of Owner’s representative and representative of Behr Process Corporation.

- Some colors, especially accent colors may require multiple finish coats for adequate coverage and opacity.

- The specified number of primer and finish coats is minimum acceptable. If full coverage and opacity is not obtained with specified number of coats, apply additional coats as necessary to produce required finish.

- Application Finish Schedule:

EXTERIOR PAINT SCHEDULE:

| Behr | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

- Stucco, Cement Plaster

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |
| Topcoat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | ESPR00 Eff-Stop Premium |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |
| Topcoat: | Premium Plus Exterior Satin 9050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | AcryShield Ext Semi-Gloss 1250 | Evershield Semi-Gloss EVSH50-2 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | AcryShield Ext Semi-Gloss 1250 | Evershield Semi-Gloss EVSH50-2 |

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Ext Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Ext Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |

- Stucco and Cement Plaster – High Build Coating

Flat – Exterior High Build Waterproof Coating, 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium High Build Coating 4700 | N/A | N/A |
| Topcoat: | Premium High Build Coating 4700 | N/A | N/A |

- Stucco, Cement Plaster, Masonry and Brick - Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric 68 | Elastakote Elastomeric 1119 | Enduralastic 10 Elastomeric EDLX10-0 |
| Topcoat: | Premium Exterior Elastomeric 68 | Elastakote Elastomeric 1119 | Enduralastic 10 Elastomeric EDLX10-0 |

4.	Concrete, Tilt-Up

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Masonry Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |

- Concrete Tilt-Up – High Build Waterproof Coating

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Flat High Build Coating 4700 | N/A | N/A |
| Topcoat: | Premium Flat High Build Coating 4700 | N/A | N/A |

- Concrete Tilt-Up – Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Primer 247 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric 68 | Elastakote Elastomeric 1119 | Enduralastic 10 Elastomeric EDLX10-0 |
| Topcoat: | Premium Exterior Elastomeric 68 | Elastakote Elastomeric 1119 | Enduralastic 10 Elastomeric EDLX10-0 |

- Concrete Masonry Units (CMU)

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | AcryShield Flat 1240 | Evershield Flat EVSH10-2 |
| Topcoat: | Premium Plus Exterior Flat 4050 | AcryShield Flat 1240 | Evershield Flat EVSH 10-2 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Exterior Satin 9050 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | AcryShield Satin 1247 | Evershield Low Sheen EVSH 40-2 |
| Topcoat: | Premium Plus Exterior Satin 9050 | AcryShield Satin 1247 | Evershield Low Sheen EVSH 40-2 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield Low Sheen EVSH 40-2 |
| Topcoat: | Premium Plus Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield Low Sheen EVSH 40-2 |

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Ext Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Ext Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |

- Concrete Masonry Units (CMU) - High Build Waterproof Coating

Flat – High Build Coating 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Flat High Build Coating 4700 | N/A | N/A |
| Topcoat: | Premium Flat High Build Coating 4700 | N/A | N/A |

- Concrete Masonry Units (CMU) - Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric 68 | Elastakote Elastomeric 1119 | Enduralastic 10 Elastomeric EDLX10-0 |
| Topcoat: | Premium Exterior Elastomeric 68 | Elastakote Elastomeric 1119 | Enduralastic 10 Elastomeric EDLX10-0 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

- Concrete Porches and Patios

Low Luster Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Low Lustre 6050 | N/A | N/A |
| Topcoat: | Premium Porch/Patio Low Lustre 6050 | N/A | N/A |

Note* Intended for use on rough porous concrete.

- Concrete Porches and Patios

Gloss Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Gloss Enamel 6700 | N/A | N/A |
| Topcoat: | Premium Porch/Patio Gloss Enamel 6700 | N/A | N/A |

Note* Intended for use on rough porous concrete.

- Concrete, Decks, Walkways, Pool Decks and Sidewalks – Solid Color Coating

Flat – Exterior Solid Color Coating, 100% Acrylic

| Prime Coat: | Behr DeckOver Solid Color Coating 5000 | N/A | N/A |
| --- | --- | --- | --- |
| Topcoat: | Behr DeckOver Solid Color Coating 5000 | N/A | N/A |

- Concrete, Decks, Walkways, Pool Decks and Sidewalks – Textured Solid Color Coating

Flat – Exterior Solid Color Coating, 100% Acrylic

| Prime Coat: | Behr Textured DeckOver Solid Color Coating 5005 | N/A | N/A |
| --- | --- | --- | --- |
| Topcoat: | Behr Textured DeckOver Solid Color Coating 5005 | N/A | N/A |

- Wood, Paint Finish

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |
| Topcoat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |
| Topcoat: | Premium Plus Exterior Satin 9050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield Semi-Gloss EVSH50-2 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield Semi-Gloss EVSH50-2 |

Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | N/A | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | N/A | Evershield Gloss EVSH60 |

(Note: Behr Premium 2-8050 Hi-Gloss Enamel conforms to VOC limits of South Coast AQMD Rule 1113.)

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | E-Z Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | 1215 Professional Semi-Gloss | Spartashield Semi-Gloss SSHL50 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | 1215 Professional Semi-Gloss | Spartashield Semi-Gloss SSHL50 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

- Wood, Paint Finish – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | EZ-Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristoshield Semi-Gloss ASHL50 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristoshield Semi-Gloss ASHL50 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | AcryShield Wood Primer 255 | EZ-Prime Premium EZPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristoshield Eggshell ASHL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristoshield Eggshell ASHL30 |

- Wood, Stain Finish, Water-Based, Semi-Transparent

Flat – 100% Acrylic

| Two Coats: | 5077 Weatherproofing Semi-Transparent Stain, Sealer | Acry-Shield Semi-Solid Stain 1285 | WPT-3 WeatherPro Stain |
| --- | --- | --- | --- |
| Or - Two Coats: | 5330 Weatherproofing Redwood Semi-Transparent Stain |  |  |
| Or – Two Coats | 5533 Weatherproofing Cedar Naturaltone Semi-Transparent Stain |  |  |

- Wood, Stain Finish, Solid Color

Flat – 100% Acrylic

| Two Coats: | 5011 Weatherproofing Solid Color Stain | Acry-Shield Semi-Solid Stain 1285 | Acri-Hues ACHS10 |
| --- | --- | --- | --- |
| Or - Two Coats: | 5013 Weatherproofing Solid Color Deep Base |  |  |
| Or – Two Coats | 50533 Weatherproofing Cedar Naturaltone Semi-Transparent Stain |  |  |

- Wood and Wood Composite – Decks, Patios, Boat Docks, Railings – Solid Color Coating

Flat – 100% Acrylic, Solid Color Coating

| Two Coats: | DeckOver Solid Color Coating 5000 | N/A | N/A |
| --- | --- | --- | --- |

- Wood and Wood Composite – Decks, Patios, Boat Docks, Railings – Textured Solid Color Coating

Flat – 100% Acrylic, Solid Color Coating

| Two Coats: | Textured DeckOver Solid Color Coating 5005 | N/A | N/A |
| --- | --- | --- | --- |

- Steel, Ferrous Metal

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |
| Topcoat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |
| Topcoat: | Premium Plus Exterior Satin 9050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield SG EVSH 50-2 |
| Topcoat: | Premium Plus Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield SG EVSH 50-2 |

Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Hi-Gloss Enamel 8050 | N/A | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Hi-Gloss Enamel 8050 | N/A | Evershield Gloss EVSH60 |

(Note: Premium Plus 2-8050 Hi-Gloss Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |

- Steel, Ferrous Metal – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristoshield Semi-Gloss ASHL50 |
| Topcoat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristoshield Semi-Gloss ASHL50 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristoshield Eggshell ASHL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristoshield Eggshell ASHL30 |

- Steel, Ferrous Metal – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat Semi-Gloss ENCT50 |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat Semi-Gloss ENCT50 |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |
| Topcoat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals

Flat – Premium Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |
| Topcoat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Flat 1240 | Evershield Flat EVSH10-2 |

Satin – Premium Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |
| Topcoat: | Premium Plus Exterior Flat 4050 | AcryShield Exterior Satin 1247 | Evershield Low Sheen EVSH40-2 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield SG EVSH 50-2 |
| Topcoat: | Premium Plus Semi-Gloss 5050 | AcryShield Semi-Gloss 1250 | Evershield SG EVSH 50-2 |

Gloss – Premium Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | N/A | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Hi-Gloss Enamel 8050 | N/A | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Hi-Gloss Enamel 8050 | N/A | Evershield Gloss EVSH60 |

(Note: Behr Premium 2-8050 Hi-Gloss Enamel conforms to VOC limits of South Coast AQMD Rule 1113.)

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Flat – Professional Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Professional Exterior Flat 1200 | Spartashield Flat SSHL10 |

Satin – Professional Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Professional Exterior Satin 1212 | Spartashield Low Sheen SSHL40 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Professional Semi-Gloss 1215 | Spartashield Semi-Gloss SSHL50 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristoshield Semi-Gloss ASHL50 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristoshield Semi-Gloss ASHL50 |

Satin – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristoshield Eggshell ASHL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristoshield Eggshell ASHL30 |

- Galvanized Metal, Aluminum, Non-Ferrous Metals – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat Semi-Gloss ENCT50 |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat Semi-Gloss ENCT50 |

- Galvanized Metal, Aluminum, Non-Ferrous Metals – High Performance, Industrial Maintenance

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | N/A | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT-60 |
| Topcoat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |

INTERIOR PAINT SCHEDULE

- Gypsum Board

Flat – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |
| Topcoat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |

Eggshell – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Satin – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |
| Topcoat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |

Semi-Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |

Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Ext/Int Gloss EVSH60 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Ext/Int Gloss EVSH60 |

(Note: Premium Plus Hi-Gloss Enamel 2-8050 conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Interior Primer 10 | AcryPlex Interior PVA Sealer 971 | Vinylastic Select VNPR00-1 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Interior Primer 10 | AcryPlex Interior PVA Sealer 971 | Vinylastic Select VNPR00-1 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Interior Primer 10 | AcryPlex Interior PVA Sealer 971 | Vinylastic Select VNPR00-1 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |

- Gypsum Board: Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | N/A | Vinylastic Select VNPR00-1 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |

- Gypsum Board – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |

Aura Grand Entrance Satin 147

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

- Gypsum Board - Ceilings

Flat – Latex, Interior

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Ceiling Flat 558 | Kel-Pro Interior Flat 705 | Acoustikote W615 |
| Topcoat: | Premium Plus Interior Ceiling Flat 558 | Kel-Pro Interior Flat 705 | Acoustikote W615 |

- Gypsum Board - Dry Fall Coating (Spray Applied)

Flat – Latex, Interior, Spray Applied

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | AcryPlex Interior PVA Sealer 971 | Vinylastic Premium VNPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White/891 Black | Dry Fog II Flat Latex 480 | Aquafall Flat W6079 |
| Topcoat: | Behr Pro Dryfall Flat 890 White/891 Black | Dry Fog II Flat Latex 480 | Aquafall Flat W6079 |

- Concrete, Plaster, Masonry

Flat – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |
| Topcoat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |

Eggshell – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |

Satin – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |
| Topcoat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |

Semi-Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |

Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |

(Note: Premium Plus 2-8050 Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |

- Gypsum Board: Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | N/A | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |

- Concrete, Plaster, Masonry – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |
| Topcoat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior PVA Sealer 971 | Eff-Stop Premium ESPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |

- Concrete Porches and Patios

Low Luster Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Low Lustre 6050 | N/A | N/A |
| Topcoat: | Premium Porch/Patio Low Lustre 6050 | N/A | N/A |

Note* Intended for use on rough porous concrete.

Gloss Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Gloss Enamel 6700 | N/A | N/A |
| Topcoat: | Premium Porch/Patio Gloss Enamel 6700 | N/A | N/A |

Note* Intended for use on rough porous concrete.

- Concrete Masonry Units (CMU)

Flat – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |
| Topcoat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |

Eggshell – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |

Satin – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |
| Topcoat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |

Semi-Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |

Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |

(Note: Premium Plus 2-8050 Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Eggshell –Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |

- Concrete Masonry Units (CMU): Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | N/A | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |

- Concrete Masonry Units (CMU) – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Semi-Gloss Enamel 3900 | Epic Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |
| Topcoat: | Alkyd Semi-Gloss Enamel 3900 | Epic Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Behr Pro Block Filler Primer 50 | Prime & Fill Block Filler 521 | Blocfil Premium SBPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Epic Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Epic Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |

- Steel, Ferrous Metal

Flat – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |
| Topcoat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |

Eggshell – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |

Satin – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |
| Topcoat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |

Semi-Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |

Gloss – Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |

(Note: Premium Plus 2-8050 Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |

- Steel, Ferrous Metal: Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | N/A | N/A |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | N/A | Bloc Rust Premium BRPR00 2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | N/A | Enduracat Semi-Gloss Pre-Catalyzed WB Epoxy ENPX50 |

- Steel, Ferrous Metal – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Alkyd Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00-2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |
| Topcoat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00-2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 3900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 3900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |

- Steel, Ferrous Metal – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | Bloc Rust Premium BRPR00-2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat ENCT60 Semi-Gloss |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat ENCT50 Semi-Gloss |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | Bloc Rust Premium BRPR00-2WH |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |
| Topcoat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals

Flat – Premium, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |
| Topcoat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

Eggshell – Premium, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |

Satin – Premium, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |
| Topcoat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |

Semi-Gloss – Premium, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |

Gloss – Premium, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | DuraPoxy Interior Gloss 1680 | Evershield Gloss EVSH60 |

(Note: Premium Plus Hi-Gloss 2-8050 Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Urethane Alkyd, Water-Reducible

Semi-Gloss – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |
| Topcoat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |

Satin – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | EPIC Int/Ext Satin 1997 | Aristowall Eggshell AWLL30 |

- Galvanized Metal, Aluminum,  Non-Ferrous Metals – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | Krud Kutter Metal Clean & Etch | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | DTM Acrylic Primer/Finish 5725 | WB Syn-Lustro Primer WSPR00 |
| Intermediate Coat: | Direct To Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat Semi-Gloss ENCT50 |
| Topcoat: | Direct To Metal Semi-Gloss 3200 | DTM Acrylic Semi-Gloss 5885 | Endura-Coat Semi-Gloss ENCT50 |

Galvanized Metal, Aluminum, Non-Ferrous Metals – High Performance, Industrial Maintenance

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Phosphoric Prep & Etch | N/A | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Multi-Surface Primer 436 | N/A | Ultra-Grip Premium UGPR00 |
| Intermediate Coat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |
| Topcoat: | Direct To Metal Gloss 8200 | N/A | Endura-Coat Gloss ENCT60 |

- Galvanized Metal and Non-Ferrous Metals – Dry Fall Coating (Spray applied)

Flat, Latex, Dry Fall

| Prime Coat: | Premium Multi-Surface Primer 436 | Kel-Bond Universal Primer 295 | Ultra-Grip Premium UGPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White/891 Black | Dry Fog II Flat Latex 480 | Aquafall Flat W6079 |
| Topcoat: | Behr Pro Dryfall Flat 890 White/891 Black | Dry Fog II Flat Latex 480 | Aquafall Flat W6079 |

- WOOD – Paint Finish, Opaque

Flat – Premium Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |
| Topcoat: | Premium Plus Interior Flat 1050 | AcryPlex Interior Flat 1602 | Everest Flat Ever10 |

Eggshell – Premium Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | AcryPlex Interior Eggshell 1610 | Everest Eggshell Ever30 |

Satin – Premium Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |
| Topcoat: | Premium Plus Interior Satin 7050 | AcryPlex Interior Satin 1640 | Suprema Satin SPMA40 |

Semi-Gloss – Premium Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | AcryPlex Int Semi-Gloss 1650 | Everest Semi-Gloss Ever50 |

Gloss – Premium Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Hi-Gloss Enamel 8050 | DuraPoxy Interior Gloss 1680 | Evershield Ext/Int Gloss EVSH60-2 |
| Topcoat: | Premium Plus Hi-Gloss Enamel 8050 | DuraPoxy Interior Gloss 1680 | Evershield Ext/Int Gloss EVSH60-2 |

(Note: Premium Plus 2-8050 Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Premium Prof. Int. Flat 1005 | SPARTAZERO Flat SZR010 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Premium Prof. Eggshell 1010 | SPARTAZERO Eggshell SZR030 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Premium Prof. Semi-Gloss 1050 | SPARTAZERO Semi-Gloss SZR050 |

| BEHR | Kelly-Moore | Dunn-Edwards |
| --- | --- | --- |

- WOOD – Paint Finish (Alkyd, Water-Reducible)

Semi-Gloss – Interior / Exterior Alkyd (Water-Reducible) Enamel

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |
| Topcoat: | Alkyd Semi-Gloss Enamel 3900 | EPIC Int/Ext Semi-Gloss 1998 | Aristowall Semi-Gloss AWLL50 |

- WOOD – Paint Finish – Dry Fall Coating (Spray applied)

Flat, Latex, Dry Fall

| Prime Coat: | Premium Plus All-In-One Primer 75 | AcryPlex Interior Undercoat 973 | Inter-Kote Premium IKPR00 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White/891 Black | Dry Fog II Flat Latex 480 | Aquafall Flat W6079 |
| Topcoat: | Behr Pro Dryfall Flat 890 White/891 Black | Dry Fog II Flat Latex 480 | Aquafall Flat W6079 |

END OF SECTION 09 90 00