SECTION 09 90 00

PAINTS and COATINGS

MASTER

ARCHITECTURAL PAINTING SPECIFICATION

(Parallel Version – PPG - BM)

The information contained in this Behr Parallel Specification is primarily intended for use in reviewing commercial painting specification submittals. Because coating manufactures may change product formulations, users of this comparative data assume all responsibilities for acceptability of references such as resin types, composition, or gloss levels contained herein. Due to the many variables of Architects’ project painting specifications, it is recommended that the paint manufacturer(s) be contacted for their product’s intended use, technical data, (M)SDS and VOC content compliance with authorities having jurisdiction.

These BEHR PAINTS are in conformance with LEED v4 BUILDING DESIGN AND CONSTRUCTION for LEED v4 BD+C New Construction, Core and Shell, Schools, Retail, Hospitality and Healthcare. All Behr Paints comply with South Coast AQMD Rule 1113. Interior and other Behr Paints are GREENGUARD® Gold Certified.

1801 E. St. Andrew Place, Santa Ana, California 92705 (714) 545-7101

**  NOTE TO SPECIFIER	This Section is for the specification of coatings in a mild to moderate environment.  It does contain some products from Diamond Vogel’s High Performance product lines to allow for design flexibility.  Diamond Vogel recommends the use of Section 09 96 00 High Performance Coatings for areas that require maximum protection.

##### PART 1 - GENERAL

##### 1.01 SUMMARY

Work includes furnishing of materials and equipment, preparation of surfaces and completion of painting and finishing of surfaces as required by Drawings and specified herein.

##### 1.02 RELATED WORK

- Factory, pre-finished items as specified in various sections.

- Shop painting specified in respective sections.

- Architectural woodworking.

- Surfaces not to be painted:

  - Pre-finished wall, ceiling, and floor coverings.

  - Items with factory-applied final finish.

  - Concealed ducts, pipes, and conduit.

  - Surfaces specifically scheduled or noted on Drawings not to be painted.

##### 1.03 SUBMITTALS

- Product data:

  - Not less than thirty (30) days before beginning work, submit a complete list of materials proposed for use, together with manufacturer's specifications.

  - Paint materials and products shall be subject to Architect's approval.

- Color samples:

  - Prepare color and finishes on samples, 8-1/2" x 11" in size.

  - Submit samples as requested until required sheen, color and texture is achieved.

  - Prepare wood samples on type and quality of wood specified for use on project.

  - Label and identify each sample as to location and application.

##### 1.04 COLORS

- Colors are to be selected or approved by Architect and actual color chips shall be supplied to Contractor for matching. All undercoats shall be tinted to approximately half the color of finish coat.

- Approval of final colors: Do not apply final coat of paint until colors have been approved by Architect.

- The number of colors to be used shall be as determined by Architect. Architect reserves right to vary colors throughout project.

##### 1.05 DELIVERY, STORAGE AND HANDLING

- Deliver paint materials in sealed original labeled containers bearing manufacturer's name, type of paint, stock number, color, and instructions for reducing or mixing where applicable.

- Paint materials and equipment

  - Store only acceptable project materials on site.

  - Store in a suitable location.

  - Restrict storage to paint materials and related materials.

  - Comply with health and fire regulations.

##### 1.06 PROJECT CONDITIONS

- Comply with manufacturer's recommendations as to environmental conditions under which coatings and coating systems can be applied. Do not apply paint or coatings when temperature is below 50F. Do not apply exterior paint in damp or rainy weather; ensure that the surface has dried thoroughly before proceeding.  Surface temperature must be at least 5°F above dew point before painting.

- Do not apply finish in areas where dust or contaminants are being generated.

#### PART 2 - PRODUCTS

##### 2.01 MATERIALS

- Basis-of-Design Product: Provide products listed from Behr Paint Company, Santa Ana, California 92705, or comparable products by one of the following. Other manufacturers to conform to materials listed and be approved by Architect.

- Sherwin-Williams Company

- Benjamin Moore & Company

- Materials selected for coating systems for each type of surface shall be the product of a single manufacturer.

- Accessory materials such as thinner, linseed oil, putty and shellac shall be of the highest quality and by approved manufacturer, which comply with local VOC content limit regulations.

- Paints shall be ready-mixed except field catalyzed coatings.

- Do not thin finish coats without Architect's approval.

- Unsuitability of specified products: Claims concerning unsuitability of any material specified or inability to satisfactorily produce the work will not be entertained, unless such claim is made in writing to Architect before work is started.

- Number of coats scheduled is minimum. Apply additional coats at no additional cost if necessary to completely hide base materials, produce uniform color and provide satisfactory finish result.

- Submitted paints and coating shall comply with current local, state, and federal air quality regulations.

#### PART 3 - EXECUTION

##### 3.01 EXAMINATION

- Examine surfaces scheduled to receive paint and finishes for conditions that will adversely affect execution, permanence, or quality of work and which cannot be put into acceptable condition through preparatory work as included in Article 3.02 "Preparation ". Notify Owner’s Representative in writing of any defects or conditions which will prevent a satisfactory installation.

- Examine surfaces scheduled to be finished prior to commencement of work. Report any condition that may potentially effect proper application.

- Maximum Moisture Content of Substrates: When measured with an electronic moisture meter as follows.

  - Concrete: 12 percent

  - Portland Cement Plaster and Stucco: 12 percent

  - Masonry (Clay and CMU): 12 percent

  - Wood: 15 percent

  - Gypsum Board: 12 percent

- Portland Cement Plaster Substrates: Verify that plaster is fully cured.

- Verify suitability of substrates, including surface conditions and compatibility with existing finishes and primers.

- Proceed with surface preparation and coating application only after unsatisfactory conditions have been corrected.

- Application of coating is construed as acceptance of surfaces and conditions.

##### 3.02 PREPARATION

- Clean surfaces to receive paint thoroughly of substances, which could impair adhesion of paints, including dust, dirt, oil, and grease before application of any coatings. Prepare surfaces as follows:

  - WOOD SUBSTRATES: Remove mill glaze and dust, sand smooth. Fill open joints, cracks, nail holes and other pits or depressions flush and smooth with wood filler after priming. Use wood putty to match finish paint coat. Touch up knots or sap streaks with a stain-blocking sealer before priming.

  - CONCRETE, PRE-CAST CONCRETE, TILT-UP: Remove release agents, curing compounds, loose particles, efflorescence, and chalk.  Do not coat surfaces if moisture content or alkalinity of surface to be coated exceeds that permitted in the manufacturer’s written instructions. Prime with an alkali-resistant primer.

  - PLASTER and STUCCO SUBSTRATES: Fill hairline cracks, small holes, and imperfections on surfaces with patching compound. Smooth off to match adjacent surfaces. Apply an alkali-resistant primer or wash with fresh water and neutralize high alkalinity surfaces where they occur.

  - MASONRY SUBSTRATES: Remove efflorescence and chalk. Do not coat surfaces if moisture content or alkalinity of surface to be coated exceeds that permitted in the manufacturer’s written instructions. Prime with an alkali-resistant primer.

  - SHOP-PRIMED STEEL SUBSTRATES: Solvent clean according to SSPC-SP1. Remove loose primer and rust, contaminants, and foreign matter. Prime bare steel and touch up abrasions with a ferrous metal primer.

  - STEEL, FERROUS METAL SUBSTRATES: Remove rust, mill scale, foreign substances, and shop primer. Clean according to SSPC-SP3, “Power Tool Cleaning”, or abrasive blasting cleaning according to SSPC-SP6 “Commercial Blast Cleaning” as required. Protect surface from corrosion until application of primer.

  - GALVANIZED METAL: Remove oils, passivators and clean entire surface with an appropriate solvent. Pre-treat with a phosphoric acid etching solution to promote adhesion of subsequently applied coatings.

  - PHOSPHATIZED METAL or GALVANNEALED METAL shall not be chemically etched. Clean and apply suitable metal primer.

  - ALUMINUM SUBSTRATES: Solvent clean according to SSPC-SP1. Remove loose surface oxidation.

  - GYPSUM BOARD SUBSTRATES: Remove dust and foreign matter. Fill pits flush and smooth with joint compound and where required, apply skim coat to provide the required finishing level based on GA-214-96, Recommended Levels of Gypsum Board Finish before application of decoration.

- Surfaces, which cannot be prepared or painted as specified, shall be immediately brought to attention of Architect in writing.

  - Starting of work without such notification will be considered acceptance by the Contractor of surfaces involved.

  - Replace unsatisfactory work caused by improper or defective surfaces as directed by Architect at no additional cost to Owner.

##### 3.03 APPLICATION

- Do not apply initial coating until moisture content of surface is within limitations recommended by paint manufacturer.

- Application:

  - Apply paint with suitable brushes, rollers or spraying equipment.

  - Apply stain in accordance with manufacturer's recommendations.

  - Rate of application shall not exceed that as recommended by paint manufacturer for surface involved.

- Comply with recommendations of product manufacturer for drying time between succeeding coats.

- Leave parts of molding and ornaments clean and true to details with no undue amount of paint in corners and depressions.

- Make edges of paint adjoining other material or color clean and sharp with no overlapping.

- Refinish whole wall where portion of finish is not acceptable.

- Apply materials evenly with proper film thickness and free of runs, sags, skips and other defects. Hard, glossy finishes shall be sanded lightly between coats, dusted, and cleaned before recoating.

- Remove hardware, hardware accessories, plates, lighting fixtures and similar items in place prior to painting and replaced upon completion of each space.

- Disconnect heating and other equipment adjacent to walls using workmen skilled in appropriate trades, and moved to permit wall surfaces to be painted. Following completion of painting, they shall be expertly replaced and reconnected.

- Paint visible surfaces behind vents, registers, or grilles flat black.

  - Wash exposed metal with solvent then prime and paint as scheduled.

  - Spray paint wherever practical.

- Do not paint over Underwriters' labels, fusible links, or sprinkler heads.

- EXPOSED PLUMBING AND MECHANICAL ITEMS: Items without factory finish such as conduits, pipes, access panels and items of similar nature shall be finished to match adjacent wall and ceiling surfaces unless otherwise directed.

##### 3.04 CLEANUP

Upon completion of work, remove equipment, excess material, and debris, remove paint splatters, and leave area in a neat and orderly condition.

##### 3.05 PROTECTION

- Protect work of other trades, whether to be painted or not, against damage by painting. Correct damage by cleaning, repairing, or replacing, and repainting, as acceptable to Architect.

- Provide "wet paint" signs to protect newly painted finishes. Remove temporary protective wrappings provided by others for protection of their work after completion of painting operations.

  - At completion of construction activities of other trades, touch up and restore damaged or   defaced painted surfaces.

##### 3.06 FINISH SCHEDULE

- Finish surfaces in accordance with schedule. Catalog names and numbers refer to products as manufactured by the Behr Paint Company, Santa Ana, California 92705, except as otherwise specified.

- Provide paint finishes of even uniform color, free from cloudy or mottled appearance. Properly correct non- complying work to satisfaction of Owner’s representative and representative of Behr Process Corporation.

- Some colors, especially accent colors may require multiple finish coats for adequate coverage and opacity.

- The specified number of primer and finish coats is minimum acceptable. If full coverage and opacity is not obtained with specified number of coats, apply additional coats as necessary to produce required finish.

- Application Finish Schedule:

EXTERIOR PAINT SCHEDULE:

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- Stucco, Cement Plaster

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Sun Proof Latex Flat 72-45XI | Moorlife Acrylic Flat N105 |
| Topcoat: | Premium Plus Exterior Flat 4050 | Sun Proof Flat Latex 72-45XI | Moorlife Acrylic Flat N105 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Latex Satin 76-45XI | Moorgard Low Lustre N103 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Latex Satin 76-45XI | Moorgard Low Lustre N103 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss 78-45XI | Moorglo Soft Gloss N096 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss 78-45XI | Moorglo Soft Gloss N096 |

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Exterior Flat 6-610XI | Ultra Spec Ext Flat N447 |
| Topcoat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Exterior Flat 6-610XI | Ultra Spec Ext Flat N447 |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2045XI | Ultra Spec Ext Satin N448 |
| Topcoat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2045XI | Ultra Spec Ext Satin N448 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-900XI | Ultra Spec Ext Gloss N449 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-900XI | Ultra Spec Ext Gloss N449 |

- Stucco and Cement Plaster – High Build Coating

Flat – Exterior High Build Coating 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Super Spec Masonry Coating 056 |
| Topcoat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Super Spec Masonry Coating 056 |

- Stucco and Cement Plaster – Texture Coating

Flat Exterior Acrylic Texture Coating – Medium

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Exterior Acrylic Texture Coating - Medium PR710 | Perma-Crete Medium Texture Coating 4-60 | High Build Exterior Texture Medium Flat 437 |
| Topcoat: | Exterior Acrylic Texture Coating -Medium PR710 | Perma-Crete Medium Texture Coating 4-60 | High Build Exterior Texture Medium Flat 437 |

- Stucco, Cement Plaster, Masonry & Brick - Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Super Spec Elastomeric N055 |
| Topcoat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Super Spec Elastomeric N055 |

- Concrete, Tilt-Up

Flat – Exterior, Professional, 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-610XI | Ultra Spec Ext Flat N447 |
| Topcoat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-610XI | Ultra Spec Ext Flat N447 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2045XI | Ultra Spec Ext Satin N448 |
| Topcoat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2045XI | Ultra Spec Ext Satin N448 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-900XI | Ultra Spec Ext Gloss N449 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-900XI | Ultra Spec Ext Gloss N449 |

- Concrete Tilt-Up – Texture Coating

Flat Exterior Acrylic Texture Coating – Medium

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XIXI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Exterior Acrylic Texture Coating - Medium PR710 | Perma-Crete Medium Texture Coating 4-60 | High Build Exterior Texture Medium Flat 437 |
| Topcoat: | Exterior Acrylic Texture Coating -Medium PR710 | Perma-Crete Medium Texture Coating 4-60 | High Build Exterior Texture Medium Flat 437 |

- Concrete Tilt-Up – High Build Coating

Flat – Exterior High Build Coating 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Super Spec Masonry Coating 056 |
| Topcoat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Super Spec Masonry Coating 056 |

- Concrete Tilt-Up – Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Perma-Crete Primer 4-603XI | Ultra Spec Masonry Sealer 608 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 C | Super Spec Elastomeric 055 |
| Topcoat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 C | Super Spec Elastomeric 055 |

- Concrete Masonry Units (CMU)

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XIXI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Sun Proof Latex Flat 72-45XI | Moorlife Acrylic Flat N105 |
| Topcoat: | Premium Plus Exterior Flat 4050 | Sun Proof Latex Flat 72-45XI | Moorlife Acrylic Flat N105 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XIXI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Latex Satin 76-45XI | Moorgard Low Lustre N103 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Latex Satin 76-45XI | Moorgard Low Lustre N103 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XIXI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss 78-45XI | Moorglo Soft Gloss N096 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss 78-45XI | Moorglo Soft Gloss N096 |

Flat – Exterior, Professional 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XIXI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-610XI | Ultra Spec Ext Flat N447 |
| Topcoat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-610XI | Ultra Spec Ext Flat N447 |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XIXI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2045XI | Ultra Spec Ext Satin N448 |
| Topcoat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2045XI | Ultra Spec Ext Satin N448 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-900XI | Ultra Spec Ext Gloss N449 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-900XI | Ultra Spec Ext Gloss N449 |

- Concrete Masonry Units – Texture Coating

Flat Exterior Acrylic Texture Coating – Medium

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Exterior Acrylic Texture Coating - Medium PR710 | Perma-Crete Medium Texture Coating 4-60 | High Build Exterior Texture Medium Flat 437 |
| Topcoat: | Exterior Acrylic Texture Coating -Medium PR710 | Perma-Crete Medium Texture Coating 4-60 | High Build Exterior Texture Medium Flat 437 |

- Concrete Masonry Units (CMU) - High Build Coating

Flat – High Build Coating 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Super Spec Masonry Coating 056 |
| Topcoat: | Premium High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Super Spec Masonry Coating 056 |

- Concrete Masonry Units (CMU) - Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Super Spec Elastomeric N055 |
| Topcoat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Super Spec Elastomeric N055 |

- Concrete Porch and Patio

Low Luster Exterior/Interior 100% Acrylic

| Prime Coat: | Concrete Bonding Primer 880* | Floor & Porch Satin 3-510XI | Latex Floor & Patio N122 |
| --- | --- | --- | --- |
| Intermediate Coat: | Porch/Patio Low Lustre Enamel 6050 | Floor & Porch Satin 3-510XI | Latex Floor & Patio N122 |
| Topcoat: | Porch/Patio Low Lustre Enamel 6050 | Floor & Porch Satin 3-510XI | Latex Floor & Patio N122 |

Note *Intended for use on rough and porous concrete surfaces.

Gloss Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | Floor & Porch Gloss 6-510C | Latex Floor & Patio Gloss 0121 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Gloss Enamel 6705 | Floor & Porch Gloss 6-510C | Latex Floor & Patio Gloss 0121 |
| Topcoat: | Premium Porch/Patio Gloss Enamel 6705 | Floor & Porch Gloss 6-510C | Latex Floor & Patio Gloss 0121 |

Note* Intended for use on rough and porous concrete surfaces.

- Anti-Slip Concrete Porch and Patio – Low Luster

Anti-Slip Low Luster Exterior/Interior 100% Acrylic

| Prime Coat: | Concrete Bonding Primer 880* | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Porch/Patio Anti-Slip Floor Paint 6250 | N/A | N/A |
| Topcoat: | Porch/Patio Anti-Slip Floor Paint 6250 | N/A | N/A |

Note *Intended for use on rough and porous concrete surfaces.

- Concrete, Decks, Walkways, Pool Decks and Sidewalks – Solid Color Coating

Flat – Premium Exterior Solid Color Coating, 100% Acrylic

| Prime Coat: | Behr Premium DeckOver 5000 | N/A | N/A |
| --- | --- | --- | --- |
| Topcoat: | Behr Premium DeckOver 5000 | N/A | N/A |

- Concrete, Decks, Walkways, Pool Decks and Sidewalks – Textured Solid Color Coating

Flat – Premium Exterior Solid Color Coating, 100% Acrylic

| Prime Coat: | Behr Textured DeckOver Solid Color Coating 5005 | N/A | N/A |
| --- | --- | --- | --- |
| Topcoat: | Behr Textured DeckOver Solid Color Coating 5005 | N/A | N/A |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- Wood, Paint Finish

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Sun Proof Exterior Flat 72-110XI | Moorlife Acrylic Flat N105 |
| Topcoat: | Premium Plus Exterior Flat 4050 | Sun Proof Exterior Flat 72-110XI | Moorlife Acrylic Flat N105 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-110XI | Moorgard Low Lustre N103 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-110XI | Moorgard Low Lustre N103 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-110XI | Moorglo Soft Gloss N096 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-110XI | Moorglo Soft Gloss N096 |

Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |

(Note: * 2-8050 Hi-Gloss Enamel conforms to VOC content limits of South Coast AQMD Rule 1113.)

Flat – Exterior, Professional 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat PR610 | Speedhide Acrylic Flat 6-650XI | Ultra Spec Ext Flat N447 |
| Topcoat: | Behr Pro e600 Exterior Flat PR610 | Speedhide Acrylic Flat 6-650XI | Ultra Spec Ext Flat N447 |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin PR640 | Speedhide Ext. Satin 6-2110XI | Ultra Spec Ext Satin N448 |
| Topcoat: | Behr Pro e600 Exterior Satin PR640 | Speedhide Ext. Satin 6-2110XI | Ultra Spec Ext Satin N448 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-901XI | Ultra Spec Ext Gloss N449 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-901XI | Ultra Spec Ext Gloss N449 |

- Wood, Paint Finish – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510XI | N/A |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510XI | N/A |

Satin – Interior / Exterior Enamel

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Seal Grip Primer & Sealer 17-921 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410XI | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410XI | Aura Grand Entrance Satin 147 |

- Wood, Stain Finish, Water-Based, Semi-Transparent

Flat – 100% Acrylic

| Two Coats: | Semi-Transparent Stain, Waterproofer | Proluxe SRD Semi-Transparent | Arborcoat WB S/T Stain 638 |
| --- | --- | --- | --- |
| Or - Two Coats: | Sealer 5077, Redwood S/T Stain 5330 | Wood Finish AKN00015 |  |
| Or – Two Coats | Cedar Naturaltone S/T Stain 5533 |  |  |

- Wood, Stain Finish, Water-Based, Solid Color

Flat – 100% Acrylic

| Two Coats: | Solid Color Stain & Waterproofer White 5011, Deep Base 5013 | Proluxe Premium Solid Color Wood Finish SIK710-100 | Arborcoat Solid Color Stain 640 |
| --- | --- | --- | --- |
| Or – Two Coats | Cedar Naturaltone S/T Stain 50533 |  |  |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- Wood, Wood Composite – Decks, Patios, Boat Docks, Railings – Solid Color Coating

Flat – 100% Acrylic, Solid Color Coating

| Two Coats: | DeckOver Solid Color Coating 5000 | N/A | N/A |
| --- | --- | --- | --- |

- Wood, Wood Composite – Decks, Patios, Boat Docks, Railings – Texture Solid Color Coating

Flat – 100% Acrylic, Solid Color Textured Coating

| Two Coats: | Advanced DeckOver Texture Coating 5005 | N/A | N/A |
| --- | --- | --- | --- |

- Steel, Ferrous Metal

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Sun Proof Exterior Flat 72-110XI | Moorlife Acrylic Flat N105 |
| Topcoat: | Premium Plus Exterior Flat 4050 | SunProof Exterior Flat 72-110XI | Moorlife Acrylic Flat N105 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-110XI | Moorgard Low Lustre W103 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-110XI | Moorgard Low Lustre W103 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-110XI | Moorglo Soft Gloss N096 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-110XI | Moorglo Soft Gloss N096 |

Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |

Flat – Exterior, Professional 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-650XI | Ultra Spec Ext Flat N447 |
| Topcoat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-650XI | Ultra Spec Ext Flat N447 |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2110XI | Ultra Spec Ext Satin N448 |
| Topcoat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2110XI | Ultra Spec Ext Satin N448 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-901XI | Ultra Spec Ext Gloss N449 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-901XI | Ultra Spec Ext Gloss N449 |

- Steel, Ferrous Metal – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Behr Interior/Exterior Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510XI | N/A |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510XI | N/A |

Satin – Interior / Exterior Enamel

| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410XI | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410XI | Aura Grand Entrance Satin 147 |

- Steel, Ferrous Metal – High Performance, Water-Based, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec DTM Semi-Gloss P29 |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec DTM Semi-Gloss P29 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Super Spec HP DTM Acrylic Gloss P28 |
| Topcoat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Super Spec HP DTM Acrylic Gloss P28 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals

Flat – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Sun Proof Exterior Flat 72-110XI | Moorlife Acrylic Flat N105 |
| Topcoat: | Premium Plus Exterior Flat 4050 | Sun Proof Exterior Flat 72-110XI | Moorlife Acrylic Flat N105 |

Satin – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-110XI | Moorgard Low Lustre W103 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-110XI | Moorgard Low Lustre W103 |

Semi-Gloss – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss 78-110Xi | Moorglo Soft Gloss N096 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss 78-110XI | Moorglo Soft Gloss N096 |

Gloss – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |

Flat – Exterior, Professional 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-650XI | Ultra Spec Ext Flat N447 |
| Topcoat: | Behr Pro e600 Exterior Flat PR 610 | Speedhide Acrylic Flat 6-650XI | Ultra Spec Ext Flat N447 |

Satin – Exterior, Professional, 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2110XI | Ultra Spec Ext Satin N448 |
| Topcoat: | Behr Pro e600 Exterior Satin PR 640 | Speedhide Ext. Satin 6-2110XI | Ultra Spec Ext Satin N448 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-901XI | Ultra Spec Ext Gloss N449 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss PR670 | Speedhide Ext. Semi-Gloss 6-901XI | Ultra Spec Ext Gloss N449 |

- Galvanized Metal, Aluminum, Non-Ferrous Metals - Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | N/A |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | N/A |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510XI | N/A |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510XI | N/A |

Satin – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410XI | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410XI | Aura Grand Entrance Satin 147 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- Galvanized Metal, Aluminum, Non-Ferrous Metals – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec HP DTM Semi-Gloss P29 |
| Topcoat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec HP DTM Semi-Gloss P29 |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Int/Ext Metal Primer 435 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer M04 |
| Intermediate Coat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Super Spec DTM Acrylic Gloss P28 |
| Topcoat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Super Spec DTM Acrylic Gloss P28 |

INTERIOR PAINT SCHEDULE

- Gypsum Board

Flat – Premium Interior, Acrylic

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-110XI | Regal Select Flat 547 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-110XI | Regal Select Flat 547 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-310XI | Regal Select Eggshell 549 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-310XI | Regal Select Eggshell 549 |

Satin – Premium Interior, Acrylic

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-3410 | Regal Select Pearl 550 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-3410 | Regal Select Pearl 550 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-510XI | Regal Select Semi-Gloss 551 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-510XI | Regal Select Semi-Gloss 551 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Advantage 900 Gloss 909-10 | Impervex Latex Gloss W309 |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Drywall Interior Primer & Sealer PX010 | Speedhide Zero Interior Latex Sealer 6-4900XI | Ultra Spec Latex Primer 534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110XI | Ultra Spec 500 Int. Flat 536 |
| Topcoat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110XI | Ultra Spec 500 Int. Flat 536 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Drywall Interior Primer & Sealer PX010 | Speedhide Zero Interior Latex Sealer 6-4900XI | Ultra Spec Latex Primer 534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |
| Topcoat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Drywall Interior Primer & Sealer PX010 | Speedhide Zero Interior Latex Sealer 6-4900XI | Ultra Spec Latex Primer 534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss 539 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss 539 |

- Gypsum Board: High Performance Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |

- Gypsum Board – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | Advance Waterborne Alkyd Semi-Gloss 793 |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | Advance Waterborne Alkyd Semi-Gloss 793 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Behr Drywall Primer & Sealer 73 | Pure Performance Primer 9-900 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |

- Gypsum Board - Ceilings

Flat – Interior, Acrylic

| Prime Coat: | Kilz PVA Drywall Primer & Sealer PX010 | Speedhide Zero Interior Latex Sealer 6-4900XI | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Interior Ceiling Flat 5558 | Ceiling Paint Flat Latex 17-45 | Muresco Ceiling White 258 |
| Topcoat: | Interior Ceiling Flat 5558 | Ceiling Paint Flat Latex 17-45 | Muresco Ceiling White 258 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- Gypsum Board - Dry Fall Coating (Spray Applied)

Flat – Interior, Acrylic, Spray Applied

| Prime Coat: | Kilz PVA Drywall Primer & Sealer PX010 | Speedhide Zero Interior Latex Sealer 6-4900XI | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro 890 White, 891 Black Dryfall Flat | Speedhide Dry-Fog Flat 6-275Xl | Sweep Up Latex Flat 153 |
| Topcoat: | Behr Pro 890 White, 891 Black Dryfall Flat | Speedhide Dry-Fog Flat 6-275Xl | Sweep Up Latex Flat 153 |

- Concrete, Plaster, Masonry

Flat – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |

Satin – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |
| Topcoat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |
| Topcoat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss 539 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss 539 |

- Concrete, Plaster, Masonry: Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |

- Concrete, Plaster, Masonry – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd Semi-Gloss 793 |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd Semi-Gloss 793 |

Satin – Interior / Exterior Enamel

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603XI | Fresh Start Latex Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Semi-Gloss 6-1410 | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Semi-Gloss 6-1410 | Aura Grand Entrance Satin 147 |

- Concrete Porches and Patios

Low Luster Exterior/Interior 100% Acrylic

| Prime Coat: | 880 Concrete Bonding Primer* | Floor & Porch Satin Latex 3-510 | Latex Floor & Patio N122 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Enamel 6050 | Floor & Porch Satin Latex 3-510 | Latex Floor & Patio N122 |
| Topcoat: | Premium Porch/Patio Enamel 6050 | Floor & Porch Satin Latex 3-510 | Latex Floor & Patio N122 |

Note* Intended for use on rough, porous concrete.

Gloss Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | N/A | Latex Floor & Patio Gloss 0121 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Gloss Enamel 6700 | N/A | Latex Floor & Patio Gloss 0121 |
| Topcoat: | Premium Porch/Patio Gloss Enamel 6700 | N/A | Latex Floor & Patio Gloss 0121 |

Note* Intended for use on rough, porous concrete.

- Concrete Masonry Units (CMU)

Flat – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |

Satin – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |
| Topcoat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |
| Topcoat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss N539 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss N539 |

- Concrete Masonry Units (CMU): Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |

- Concrete Masonry Units – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd 793 |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd 793 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Block Filler 160 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Semi-Gloss 6-1410 | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Semi-Gloss 6-1410 | Aura Grand Entrance Satin 147 |

- Steel, Ferrous Metal

Flat – Premium Interior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat K547 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat K547 |

Eggshell – Premium Interior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |

Satin – Premium Interior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Semi-Gloss – Premium Interior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Eggshell 9-300Xl | Regal Select Semi-Gloss 551 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Eggshell 9-300Xl | Regal Select Semi-Gloss 551 |

Gloss – Premium Interior 100% Acrylic

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer p04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Pure Performance Semi-Gloss 9-500Xl | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Pure Performance Semi-Gloss 9-500Xl | Impervex Latex Gloss W309 |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |
| Topcoat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |
| Topcoat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss N539 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss N539 |

- Steel, Ferrous Metal – Light Industrial Coatings

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Multi-Surface Primer & Sealer 436 |  | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |  |
| --- | --- | --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy |  |  | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy |  |  | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Multi-Surface Primer & Sealer 436 |  | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |  |
| --- | --- | --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy |  |  | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy |  |  | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |

- Steel, Ferrous Metal – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Premium Interior / Exterior Enamel

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | Advance Waterborne Semi-Gloss Alkyd 793 |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | Advance Waterborne Semi-Gloss Alkyd 793 |

Satin – Premium Interior / Exterior Enamel

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |

| BEHR | Sherwin-Williams | Benjamin Moore |
| --- | --- | --- |

- Steel, Ferrous Metal – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec HP DTM Semi-Gloss P29 |
| Topcoat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec HP DTM Semi-Gloss P29 |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Super Spec HP DTM Acrylic Gloss P28 |
| Topcoat: | Premium Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Super Spec HP DTM Acrylic Gloss P28 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals

Flat –Premium Interior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |

Eggshell – Premium Interior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Regal Select Eggshell 549 |

Satin – Premium Interior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |

Semi-Gloss – Premium Interior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |

Gloss – Premium Interior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |
| Topcoat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |
| Topcoat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15XI | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss N539 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss N539 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Light Industrial Coatings

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Premium Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd 793 |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd 793 |

Satin – Premium Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Urethane Alkyd Satin 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |

- Galvanized Metal, Aluminum,  Non-Ferrous Metals – High Performance, Industrial Maintenance

Semi-Gloss, Premium 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec DTM Acrylic Semi-Gloss P29 |
| Topcoat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Super Spec DTM Acrylic Semi-Gloss P29 |

Gloss, Premium 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch (GKPA30220) | None recommended. | Krud Kutter Metal Clean & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| Intermediate Coat: | Premium Direct-To-Metal Gloss 8200 | 90-1310 Pitt-Tech Plus Gloss DTM | Super Spec DTM Acrylic Gloss P28 |
| Topcoat: | Premium Direct-To-Metal Gloss 8200 | 90-1310 Pitt-Tech Plus Gloss DTM | Super Spec DTM Acrylic Gloss P28 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- Galvanized Metal and Non-Ferrous Metals – Dry Fall Coating (Spray applied)

Flat, Acrylic / PVA, Dry Fall

| Prime Coat: | Multi-Surface Primer & Sealer 436 | Pitt-Tech Plus Primer 90-912 | Super Spec Metal Primer P04 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | Sweep Up Latex Flat 153 |
| Topcoat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | Sweep Up Latex Flat 153 |

- WOOD – Paint Finish

Flat – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Regal Select Flat 547 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | 9-300XI Pure Performance Eggshell | Regal Select Eggshell 549 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | 9-300XI Pure Performance Eggshell | Regal Select Eggshell 549 |

Satin – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Regal Select Pearl 550 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Regal Select Semi-Gloss 551 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss2-8050 | Manor Hall Gloss 52-110 | Impervex Latex Gloss W309 |

(Note: 2-8050 Hi-Gloss Enamel conforms to VOC limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |
| Topcoat: | Behr Pro i300 Interior Flat PR310 | Speedhide Zero Interior Flat 6-4110 | Ultra Spec 500 Int. Flat 536 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |
| Topcoat: | Behr Pro i300 Interior Eggshell PR330 | Speedhide Zero Interior Eggshell 6-4310XI | Ultra Spec 500 Eggshell 538 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss 539 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss PR370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | Ultra Spec 500 Semi-Gloss 539 |

| BEHR | PPG Paints | Benjamin Moore |
| --- | --- | --- |

- WOOD – Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V342 Pre-Catalyzed Waterborne Epoxy Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Corotech V341 Pre-Catalyzed Waterborne Epoxy Semi-Gloss |

- WOOD – Paint Finish – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Premium Interior / Exterior Alkyd (Water-Reducible) Enamel

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd 793 |
| Topcoat: | Urethane Alkyd Semi-Gloss 3900 | Speedhide WB Alkyd SG 6-1510 | Advance Waterborne Alkyd 793 |

Satin – Premium Interior / Exterior Alkyd (Water-Reducible) Enamel

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Fresh Start Primer N023 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |
| Topcoat: | Urethane Alkyd Satin 7900 | Speedhide WB Alkyd Satin 6-1410 | Aura Grand Entrance Satin 147 |

- WOOD – Paint Finish – Dry Fall Coating (Spray applied)

Flat, Acrylic / PVA, Dry Fall

| Prime Coat: | Premium All-In-One Primer 75 | Pure Performance Primer 9-900 | Ultra Spec Latex Primer N534 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | 153 Sweep Up Latex Flat |
| Topcoat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | 153 Sweep Up Latex Flat |

END OF SECTION 09 90 00